# Embedded Systems Retrieval Practice Worksheets

Worksheet 1 – Digital I/O Fundamentals

Topic: Configuring and controlling input/output pins on ATmega328

Part A – Concept Recall (Answer without using notes)

1. What is the function of each of these registers:

a) DDRx

b) PORTx

c) PINx

2. How do you configure PB0 as output and PD2 as input?

3. What happens if you try to write PORTB |= (1<<PB0); without first setting DDRB?

4. A push button is connected between PD2 and GND. Internal pull-up is enabled.

- What will be the logic level of PD2 when the button is not pressed?

- What happens when you press the button?

5. Explain what each line of the code below does:

DDRB |= (1<<PB0);

PORTB &= ~(1<<PB0);

Part B – Predict the Output

1. Predict the LED behavior:

DDRB |= (1<<PB0);

while(1){

PORTB ^= (1<<PB0);

_delay_ms(1000);

}

2. What will happen if you replace ^= with |= ?

Part C – Short Coding Task

Write a small program:

- LED on PB0 should stay ON only when the button on PD2 is pressed.

- Use internal pull-up resistor.

Skill built: Hardware control logic + register thinking

Next ready topic: Timer and delay mechanisms

## Worksheet 2 – Timers and Delay Control

Topic: Using timer overflow and delays to control execution

Part A – Recall

1. What is the purpose of TCNT0 and TCCR0B registers?

2. What does the prescaler do?

3. Name the bit in TIFR0 that indicates a timer overflow.

4. Why should we clear this flag manually in software?

Part B – Predict

1. What happens if you keep TCNT0 = 0 and set prescaler to 1024?

2. If overflow flag is not cleared, what behavior will you observe?

3. Predict the blink speed difference when prescaler is 64 vs 1024.

Part C – Mini Task

Modify the LED blink program so that:

- The LED toggles only when Timer0 overflows.

- Use polling (no interrupts yet).

(Hint: Wait for TOV0 bit to become 1, then clear it and toggle LED.)

Skill built: Polling and flag checking - directly applicable to UART transmit-ready flags.

## Worksheet 3 – PWM and Analog Output Control

Topic: Generating PWM signals for LED brightness control

Part A – Recall

1. Which mode enables PWM generation in Timer0?

2. Which register stores the PWM duty cycle value?

3. What happens when OCR0A = 0 vs OCR0A = 255?

4. Explain “duty cycle” in your own words.

5. Which pins support PWM output on ATmega328?

Part B – Predict

1. Predict LED brightness when OCR0A = 128.

2. What happens if you forget to set DDRD |= (1<<PD6)?

3. Predict the change in brightness if OCR0A is increased gradually in a loop.

Part C – Mini Task

Write a program to fade an LED connected to PD6:

- Increase brightness from 0 to max.

- Then decrease back to 0.

- Use _delay_ms(10) between steps.

(Bonus: Try without using delay, by counting timer overflows instead.)

Skill built: Continuous signal generation and register familiarity